Project #1

01/30/2026 |

CIVE 202- Civil analysis

Emma Robbins

2026

Project #1: Organizing and Summarizing Air Quality Data

Client: UNMC Water, Climate, and Health Working Group

Prepared by: Emma Robbins

Introduction/Background

The University of Nebraska Medical Center (UNMC) Water, Climate, and Health groups are working with health professionals and environmental scientists to make an effort to monitor air quality conditions based on the AirPurple sensor network. UNMC specializes in the health impacts of pollution, and this project was established to bridge the technical gap.

The goal of this project is to analyze the air quality dataset from February 2024 to March 2025. The objective is to transform the data into insights regarding Volatile Organic Compounds (VOCs) and Particulate Matter (PM2.5 and PM10.0) across Nebraska’s landscape.

## Scope of Work

### Project Goals

The goal of this project is to analyze the air quality data set from the AirPurple monitors from February 2024 to March 2025. From the provided data set, several key findings will be determined, which will be reported back to UNMC. First, the project will enable UNMC to determine if the air quality data collected meets the EPA National Ambient Air Quality Standards (NAAQS). Second, is locating the areas with the highest average and median concentrations of VOC, PM2.5, and PM10.0. Third, includes evaluating how temperature, humidity, geographical location, and elevation influence the air quality. Finally, identify the “hotspots” that may impact the public and cause concerns.

### Project Tasks

#### Task 1: Locations with the highest mean and median concentrations of VOC, PM2.5, and PM10.0.

    - The first task of this project focuses on building a baseline through the code by identifying the mean and median concentrations for Volatile Organic Compounds (VOC), PM2.5, and PM10.0 to identify the five locations exhibiting the highest pollutant density. To ensure the data is filtered properly, I will be utilizing Python to first filter the data based on locations, then take the mean and median values of the data, and then separate the data to show the 5 locations with the highest values of mean and median values. This code will pinpoint the locations where the highest concentrations have occurred.

#### Task 2: Daily maximum values including locations.

    - Task two will examine the relationship between the values recently exported in the prior task and the environmental conditions that occurred during the exact dates and geographical locations. Python will allow the data to filter out the maximum values for VOC, Pm2.5, and PM10.0 based on their specific days. I will also use Python to display where these peaked values occurred. After the days are exported, prior research will tell whether the values had to deal with weather conditions, seasonal trends, and localized emission sources to explain why those values are peaked.

#### Task 3: Impact of temperature and humidity.

    - This part of the project examines the possible effect of environmental factors on air quality. The humidity levels will be separated into Low(<50%), Medium (50-80%, and High (>80% while temperature data will be placed in four categories: Below Freezing (<32 degrees), Cool (32-50 degrees), (Warm (51-70 degrees), and Hot (>70 degrees). By performing detailed data set comparisons and utilizing graphical analysis, it will become clear if these specific conditions correlate with shifts in pollutant concentrations.

#### Task 4: Identification of Air Quality Index health risks at specific locations.

    - The last part of the project will translate the pollutant data into public health insights by converting PM2.5 and PM10.0 into standard EPA Air Quality Index (AQI) categories. Using the EPA reference tables and data comparison, the code isolates and identifies when and where air quality levels were “Unhealthy for Sensitive Groups” or higher. The event is documented to include the specific location of the sensor and the day it was recorded. This analysis will conclude by referencing the data to potential environmental factors caused for these health risk events.

## Methods

Using the air quality data provided, there were several steps taken to make this project complete: data preprocessing and cleaning, statistical analysis, meteorological analysis, and technical reproducibility.

Data Preprocessing and Cleaning

To ensure the data would work correctly, there were a few steps that were done using the Python library. The code imports the “pandas package using the import function. Then, the CSV of air quality was imported into the Python environment. The code groups the date and the location. The code outputs the top 5 mean VOC values. The next code does the same for PM2.5 and PM10.0. The median, mean, date, and location are printed to display the values. The next step was to customize the rulebook and use conditional logic to decide what labels are assigned to numbers. This was applied to the temperature and humidity column, which then printed the correct label to the corresponding value for the first five rows of the table. The final action was to determine how many values from the PM2.5 and PM10.0 columns were unhealthy for sensitive groups. The PM2.5 values fall between 35.5 and 55.4, and the PM10.0 falls between 155 and 254 (Redirecting, n.d.). The code looks through the whole spreadsheet and prints the date, location name, and either PM2.5 or PM10.0. This turns the list into a timeline to see if the unhealthy events happen all at once or over the month.

## Deliverables

#### This project will include several different files to present the data to its entirety. The project will include:

    - Raw data files used in the Python Code file

    - Python code file that documents the code used to generate the solution.

    - A link to the projects GitHub, which includes the Python code, csv with data, and the README file.

    - A scope of work to relate to the client prompt.

    - A written report that summarizes the findings contained in the code meets the requests of the client.

## Results

#### Pollution Hotspots in Nebraska

| VOC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| date |  |  | sensor.name | mean |  |  | median |  |  |
| 6/24/2024 |  |  | Swnphd-ogallala | 1209.931571 |  |  | 1209.93157 |  |  |
| 10/5/2024 |  |  | SWNPHD-Imerial | 1135.473 |  |  | 1135.473 |  |  |
| 11/23/2024 |  |  | SWNPHD-Imerial | 1041.722 |  |  | 1041.722 |  |  |
| 10/12/2024 |  |  | ELVPHD Norfolk HD 4 | 884.649 |  |  | 884.649 |  |  |
| 10/16/2024 |  |  | SWNPHD-Imerial | 871.182 |  |  | 871.182 |  |  |
| PM2.5 |  |  |  |  |  |  |  |  |  |
| date |  | sensor.name |  |  | mean |  |  | median |  |
| 2/18/2025 |  | #16 - Richardson County Courthouse |  |  | 3782.823313 |  |  | 3782.82331 |  |
| 2/19/2025 |  | #16 - Richardson County Courthouse |  |  | 3401.999333 |  |  | 3401.99933 |  |
| 2/20/2025 |  | #16 - Richardson County Courthouse |  |  | 3243.746646 |  |  | 3243.74665 |  |
| 1/21/2025 |  | #16 - Richardson County Courthouse |  |  | 3209.817146 |  |  | 3209.81715 |  |
| 2/18/2025 |  | Broken Bow |  |  | 3205.868854 |  |  | 3205.86885 |  |
| PM10.0 |  |  |  |  |  |  |  |  |  |
| date | sensor.name |  |  |  |  | mean |  |  | median |
| 2/18/2025 | #16 - Richardson County Courthouse |  |  |  |  | 3784.682542 |  |  | 3784.68254 |
| 2/19/2025 | #16 - Richardson County Courthouse |  |  |  |  | 3403.435958 |  |  | 3403.43596 |
| 2/20/2025 | #16 - Richardson County Courthouse |  |  |  |  | 3245.613354 |  |  | 3245.61335 |
| 1/21/2025 | #16 - Richardson County Courthouse |  |  |  |  | 3211.046813 |  |  | 3211.04681 |
| 2/18/2025 | Broken Bow |  |  |  |  | 3206.779708 |  |  | 3206.77971 |

Looking at the tables above, present the 5 locations with the highest mean and median concentrations of VOC, PM2.5, and PM10.0. These tables also present on what days the maximum values occur and where they occur.

Based on previous research from outside sources on the locations and dates when these maximum values occur, we can draw a few different conclusions.

#### Peak Event Analysis

VOC Data

The VOC values are spiked by human activity and temperature. These maximum values are recorded during the summer months, when higher temperatures increase the VOC emissions, and the sunlight helps VOCs accumulate instead of dispersing. The locations where the maximums were recorded are in areas with large amounts of traffic. Ogallala is a town located near the interstate, where the VOCs will be elevated from the vehicle exhaust and fuel evaporation. The sensors are near urban areas with much higher max VOCs than rural or elevated sites.

PM2.5

The PM2.5 values can be increased from calm or stagnant air, temperature inversions, or specific emission events. These locations where the maximum values were recorded are in very low elevations and enclosed areas. These types of locations trap fine particles more easily, making the PM2.5 values increase. The dates of these recorded values respond strongly to short-term weather changes. These values were recorded in January where weather changes at the drop of a hat. One bad atmosphere day can create the highest value of the month.

PM1.0

PM10.0 values depend a lot on dry conditions, windy days, and locations near open land or construction. Broken Bow is located in a very open landscape with more open ground and roadways. This will show higher PM10 values. The Richardson County Courthouse is also located in open ground and roadways. These values are taken during the winter months, when it is windy and dry, causing a spike in PM10 values.

#### Meteorological Impacts

The temperature does affect air quality, with higher temperatures corresponding with increased VOC values, and in certain locations, it raises PM2.5 and PM10 values. The humidity doesn't have as strong an influence on the air quality; the humidity levels remained low and didn't show much variation. Overall, the temperature is a larger factor affecting air quality than humidity for the time period and locations presented.

#### AQI and Public Health Risks

During the winter, the temperature increases because a layer of warm air can trap cold air and the pollutants within it close to the ground. During the holidays, as well, there is an increase in wood-burning for heating homes. This and the combinations of stagnant air led to spikes in PM2.5.

During the summer months, there is a higher risk of wildfires and agricultural burning. The smoke from these fires can cause a spike in the PM1.0. This increases these risk unhealthy air quality for sensitive groups. This would explain why people with seasonal allergies struggle to breathe so much when there is a nearby fire.

## References

API. (n.d.). PurpleAir Community. https://community.purpleair.com/c/data/api/18

AirNow.gov. (n.d.). https://www.airnow.gov/?city=false&country=false

https://uofnebraska-my.sharepoint.com/personal/37381171_nebraska_edu/_layouts/15/onedrive.aspx?id=%2Fpersonal%2F37381171%5Fnebraska%5Fedu%2FDocuments%2FTeaching%2FCIVE%20202%2FSpring%202026%2FProject%20%231%2FEPA%20Air%20Quality%20Index%20Document%2Epdf&parent=%2Fpersonal%2F37381171%5Fnebraska%5Fedu%2FDocuments%2FTeaching%2FCIVE%20202%2FSpring%202026%2FProject%20%231&ga=1

US EPA PM2.5 Air Quality Standards Interactive Map by PurpleAir. (n.d.). PurpleAir. https://map.purpleair.com/air-quality-standards-us-epa-aqi?opt=%2F1%2Flp%2Fa10%2Fp604800%2FcC0#5.21/40.002/-97.008